Crystal Structure Report for Prof. Kovacs

Submitted by Rose Theisen

Sample ID – RMT-2-91

by Antonio DiPasquale

X-ray Crystallography Laboratory

University of Washington

29. July 2026

Data backup accessibly through ftp:

computer: 		chmraypc.cpac.washington.edu

login: 		anonymous

password:		(your name or email address)

location: 	c:\structures\

Data backup accessibly through http:

URL: 	http://cad4.cpac.washington.edu/structures

Username: 	xray-user

Password: 	hkl-data

A.DiPasquale:(206) 616-4212

(x-ray lab):     (206) 543-0210

Email:	adipasqu@u.washington.edu

Understanding crystallographic information is not trivial, if you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.  For many structures, it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even we have to pay for our own structures for our research.

A orange crystal prism 0.24 x 0.12 x 0.07 mm in size was mounted on a glass capillary with oil.  Data was collected at -143oC with one set of exposure as follows:

Scan 		Type	Scanlength (deg.)  start (deg.):	     		     	

1	      	        	183.0			          	         3.703    -1.5      160    0.000

Crystal-to-detector distance was 30 mm and exposure time was 60 seconds per degree for all sets.  The scan width was 1.0o.   Data collection was 75.8% complete to 28.31o in .  A total of 63,100 partial and complete reflections were collected covering the indices, h = -13 to 11, k = -18 to 14,

l = -20 to 16.  8,585 reflections were symmetry independent and the Rint = 0.0834 indicated that the data was of slightly worse than average quality (0.07).

Indexing and unit cell refinement indicated a triclinic P lattice.  The space group was found to be P -1 (No.2).

The data was integrated and scaled using hkl-2000. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S is calculated from the scale and the B factor, which is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SHELXS-97) produced a complete heavy atom phasing model consistent with the proposed structure.

All hydrogen atoms were placed using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares (SHELXL-97).