Crystal Structure Report for J Mayer
by Werner Kaminsky

Sample Submitted by Alex Fox 

ID – af001
X-ray Crystallography Laboratory

University of Washington

09. January 2011
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@chem.washington.edu
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A colorless plate, measuring 0.19 x 0.10 x 0.08 mm3 was mounted on a glass capillary with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation.
Crystal-to-detector distance was 40 mm and exposure time was 60 seconds per degree for all sets.  The scan width was 0.5o.   Data collection was 99.6% complete to  in (.  A total of  73552 (merged) reflections were collected covering the indices, h = -17 to 17, k = -19 to 19,  l = -22 to 22.  14527 reflections were symmetry independent and the Rint = 0.1717 indicated that the data was problematic (average quality 0.07).  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be P(1  (No.2). 

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.
 

Solution by direct methods (SHELXS, SIR97
) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97.
,
 Scattering factors are from Waasmair and Kirfel
. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.
The unit cell consists of two porphyrin molecules and two anions. In addition were found two disordered toluene molecules, one pentane, one disordered ethanol, coordinating through the lone pair with the iron of one porphyrin and fourfold disordered ether coordinating similarly with iron of the other. The general disorder is best visualized by the asymmetric unit which shows two half porphyrins and several half-site occupancy solvent molecules. Inversion symmetry completes the porphyrins and produces a second set of solvent molecules through inversion at the Fe centers.
The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP
 of the asymmetric unit.
[image: image1.jpg]



Figure 1. ORTEP of one porphyrin of the structure with thermal ellipsoids at the 50% probability level. Disorder and solvent omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Identification code 
af001

Empirical formula 
C185 H148 B2 F40 Fe2 N8 O2

Formula weight 
3408.43

Temperature 
100(2) K

Wavelength 
0.71073 Å

Crystal system 
Triclinic

Space group 
P(1
Unit cell dimensions
a = 14.4208(2) Å
= 103.1970(10)°.


b = 16.3043(2) Å
= 107.3590(10)°.


c = 18.3540(2) Å
 = 91.8310(10)°.

Volume
3986.77(9) Å3
Z
1

Density (calculated)
1.420 Mg/m3
Absorption coefficient
0.288 mm-1
F(000)
1752

Crystal size
0.19 x 0.10 x 0.08 mm3
Theta range for data collection
1.49 to 25.35°.

Index ranges
-17<=h<=17, -19<=k<=19, -22<=l<=22

Reflections collected
73552

Independent reflections
14527 [R(int) = 0.0324]

Completeness to theta = 25.00°
99.6 % 

Max. and min. transmission
0.9773 and 0.9473

Refinement method
Full-matrix least-squares on F2
Data / restraints / parameters
14527 / 97 / 1300

Goodness-of-fit on F2
1.012

Final R indices [I>2sigma(I)]
R1 = 0.0581, wR2 = 0.1469

R indices (all data)
R1 = 0.0874, wR2 = 0.1732

Largest diff. peak and hole
0.610 and -0.429 e.Å-3



� Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS


(version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.


� (a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.


� Sheldrick, G. M. SHELXL-97: Program for the Refinement of Crystal Structures 1997 University of Gottingen, Germany.


� Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.


� Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.


� Farrugia, L. J. Appl. Cryst. 1997, 30, 565





