Crystal Structure Report for Julie Kovacs
by M. Coggins and W. Kaminsky
Sample Submitted by Ben Leipzig 

ID – Ben10
X-ray Crystallography Laboratory

University of Washington

06. January 2012
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@chem.washington.edu
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A red prism, measuring 0.20 x 0.10 x 0.10 mm3 was mounted on a glass capillary with oil.  Data was collected at -173 oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation.
Crystal-to-detector distance was 40 mm and exposure time was 10 seconds per degree for all sets.  The scan width was 0.5o.   Data collection was 100% complete to 25o  in (.  A total of  226952 (merged) reflections were collected covering the indices, h = -25 to 25, k = -22 to 22,  l = -29 to 29.  18189 reflections were symmetry independent and the Rint = 0.1136 indicated that the data was of less than average quality (0.07).  Indexing and unit cell refinement indicated a primitive monoclinic lattice.  The space group was found to be P  21/n (No.14). 

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.
 

Solution by direct methods (SHELXS, SIR97
) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97.
,
 Scattering factors are from Waasmair and Kirfel
. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.
The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP
 of the asymmetric unit.
[image: image1.png]



Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. 
Table 1: Crystallographic data for the structures provided.

Empirical formula 
C74 H94 Cl2 Co2 N8 O6 P2 S2

Formula weight 
1506.39

Temperature 
100(2) K

Wavelength 
0.71073 Å

Crystal system 
Monoclinic

Space group 
P 21/n
Unit cell dimensions
a = 19.2567(12) Å
= 90°.


b = 16.7383(11) Å
= 92.034(3)°.


c = 22.2858(13) Å
 = 90°.

Volume
7178.7(8) Å3
Z
4

Density (calculated)
1.394 Mg/m3
Absorption coefficient
0.698 mm-1
F(000)
3168

Crystal size
0.20 x 0.10 x 0.10 mm3
Theta range for data collection
1.37 to 28.49°.

Index ranges
-25<=h<=25, -22<=k<=22, -29<=l<=29

Reflections collected
226952

Independent reflections
18025 [R(int) = 0.1136]

Completeness to theta = 25.00°
100.0 % 

Max. and min. transmission
0.9335 and 0.8731

Refinement method
Full-matrix least-squares on F2
Data / restraints / parameters
18025 / 0 / 875

Goodness-of-fit on F2
1.052

Final R indices [I>2sigma(I)]
R1 = 0.0532, wR2 = 0.1156

R indices (all data)
R1 = 0.0992, wR2 = 0.1382

Largest diff. peak and hole
0.616 and -0.657 e.Å-3
� Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS


(version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.


� (a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.


� Sheldrick, G. M. SHELXL-97: Program for the Refinement of Crystal Structures 1997 University of Gottingen, Germany.


� Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.


� Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.


� Farrugia, L. J. Appl. Cryst. 1997, 30, 565





