Crystal Structure Report for 

Satheeshkumar Rajendran
by Werner Kaminsky

ID – ClAHMHH
X-ray Crystallography Laboratory

University of Washington

12. August 2014
Data Collection  started: 8/7/2014
Documentation finished: 8/12/2014
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@chem.washington.edu
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A colorless prism, measuring 0.35 x 0.25 x 0.23 mm3 was mounted on a loop with oil.  Data was collected at -143oC on a Nonius Kappa CCD FR590 single crystal X-ray diffractometer, Mo-radiation.
Crystal-to-detector distance was 30 mm and exposure time was 30 seconds per degree for all sets.  The scan width was 2o.   Data collection was 99.8% complete to 25o  in (.  A total of  6290 merged reflections were collected covering the indices, h = -12 to 12, k = -11 to 12,  l = -14 to 14.  12420 reflections were symmetry independent and the Rint = 0.0423 indicated that the data was good.  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be P(1  (No.2). 

The data was integrated and scaled using hkl-SCALEPACK.
 This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SHELXS, SIR97
) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97.
,
 Scattering factors are from Waasmair and Kirfel
. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.
The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP
 of the asymmetric unit.
[image: image2.png]Y

| \ais
SR

on c4 (4'/ \ .
~os \L/ \‘5/\/< 2

oL ]
N /Iru\ AN —

&3

cis




Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. 
Table 1: Crystallographic data for the structures provided.

Empirical formula 
C20 H14 Cl N3

Formula weight 
331.79

Temperature 
130(2) K

Wavelength 
0.71073 Å

Crystal system 
triclinic
Space group 
P -1
Unit cell dimensions
a = 9.0854(6) Å
= 115.464(4)°.


b = 9.4850(10) Å
= 91.379(6)°.


c = 10.8016(11) Å
 = 106.078(6)°.

Volume
796.46(13) Å3
Z
2

Density (calculated)
1.384 Mg/m3
Absorption coefficient
0.245 mm-1
F(000)
344

Crystal size
0.30 x 0.25 x 0.23 mm3
Theta range for data collection
2.44 to 28.29°.

Index ranges
-12<=h<=10, -11<=k<=12, -14<=l<=14

Reflections collected
6290

Independent reflections
3886 [R(int) = 0.0423]

Completeness to theta = 25.00°
99.8 % 

Max. and min. transmission
0.9458 and 0.9301

Refinement method
Full-matrix least-squares on F2
Data / restraints / parameters
3886 / 0 / 220

Goodness-of-fit on F2
1.025

Final R indices [I>2sigma(I)]
R1 = 0.0461, wR2 = 0.1217

R indices (all data)
R1 = 0.0639, wR2 = 0.1325

Largest diff. peak and hole
0.323 and -0.438 e.Å-3
� Otwinowsky, Z.; Minor, W. In: Methods in Enzymology; Carter, C. W., Jr., Sweet, R. M., Eds.; Academic Press: New York, 1997, 276, 307-326.


� (a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.


� Sheldrick, G. M. SHELXL-97: Program for the Refinement of Crystal Structures 1997 University of Gottingen, Germany.


� Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.


� Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.


� Farrugia, L. J. Appl. Cryst. 1997, 30, 565





_1097643391.unknown

