Crystal Structure Report for Julie Kovacs

Submitted by Santiago Toledo

Sample ID – Fe2tf

by Jason B. Benedict

and Erica Gunn

X-ray Crystallography Laboratory

University of Washington

01. May 2006
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@wintensor.com
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A clear crystal plate cut down to 0.24 x 0.24 x 0.10 mm was mounted on a glass capillary with oil.  Data was collected at -143oC with one set of exposures as follows:

Scan 

Type
Scanlength (deg.)  start (deg.)
(     
(
(     
( 

1
      
(        
182.0




3.70
-1.5
160  
 0 

Crystal-to-detector distance was 30 mm and exposure time was 30 seconds per degree for all sets.  The scan width was 1.0o.   Data collection was 91.0% complete to 28.31o and 97.1% complete to 25o  in (.  A total of 22662 partial and complete reflections were collected covering the indices, h = -10 to 10, k = -10 to 10, l = -35 to 35.  4124 reflections were symmetry independent and the Rint = 0.0458 indicated that the data was of better than average quality (0.07). 

Indexing and unit cell refinement indicated a monoclinic P lattice.  The space group was found to be P 21/c (No.14). 

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. 

All hydrogen atoms were located using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares. 

Far from the proposed structure, this compound appears to be the result of a type of acetonitrile polymerization. Counting atoms, one can deduce six individual CH3CN molecules reacted to form the molecule shown on the right. This molecule carries a +1 charge, delocalized around the two protonated nitrogens, to balance the negative charge on the triflate anion. The assignment of these proton positions agrees with fourier differences maps, additionally providing favorable hydrogen bonding geometries with oxygens in the triflate anion.




_1097643391.unknown

