Crystal Structure Report for G. Lalic
by Sarah Flowers and W. Kaminsky
Sample Submitted by Hester Dang 

ID – Hester_BAM
X-ray Crystallography Laboratory

University of Washington

15. September 2014
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Data Collection  started: 8/27/2014

Documentation finished: 9/15/2014

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@chem.washington.edu
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A yellow prism, measuring 0.32 x 0.03 x 0.03 mm3 was mounted on a glass capillary with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation.
Crystal-to-detector distance was 40 mm and exposure time was 240 seconds per degree for all sets.  The scan width was 1.0o.   Data collection was 99.8% complete to 24.76o.  A total of  37,909 (merged) reflections were collected covering the indices, h = -28 to 28, k = -7 to 7,  l = -29 to 29.  3,300 reflections were symmetry independent and the Rint = 0.141 indicated that the data was less than average quality (average quality 0.07).  Indexing and unit cell refinement indicated an orthorhombic lattice.  The space group was found to be P b c a (No.61). 

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.
 

Solution by direct methods (SHELXS, SIR97
) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97.
,
 Scattering factors are from Waasmair and Kirfel
. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.
A close bond distance between O4 and itself related through symmetry of 2.60 Å is noticed.  This is due to hydrogen bond interactions with the hydrogen of  N3. 
The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP
 of the asymmetric unit.
[image: image1.png]



Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. 
Table 1.  Crystal data and structure refinement for hd_bam_0ma.

Empirical formula 
C21 H16 F2 N4 O4

Formula weight 
426.38

Temperature 
100(2) K

Wavelength 
0.71073 Å

Crystal system 
Orthorhombic

Space group 
P b c a

Unit cell dimensions
a = 24.4634(19) Å
= 90°.


b = 6.1982(4) Å
= 90°.


c = 25.443(2) Å
 = 90°.

Volume
3857.8(5) Å3
Z
8

Density (calculated)
1.468 Mg/m3
Absorption coefficient
0.117 mm-1
F(000)
1760

Crystal size
0.32 x 0.03 x 0.03 mm3
Theta range for data collection
1.80 to 24.76°.

Index ranges
-28<=h<=28, -7<=k<=7, -29<=l<=29

Reflections collected
37909

Independent reflections
3300 [R(int) = 0.1414]

Completeness to theta = 24.76°
99.8 % 

Absorption correction
Semi-empirical from equivalents

Max. and min. transmission
0.9965 and 0.9636

Refinement method
Full-matrix least-squares on F2
Data / restraints / parameters
3300 / 0 / 281

Goodness-of-fit on F2
1.003

Final R indices [I>2sigma(I)]
R1 = 0.0487, wR2 = 0.0925

R indices (all data)
R1 = 0.1047, wR2 = 0.1112

Largest diff. peak and hole
0.283 and -0.254 e.Å-3
� Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.


� (a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.


� Sheldrick, G. M. SHELXL-97: Program for the Refinement of Crystal Structures 1997 University of Gottingen, Germany.


� Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.


� Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.


� Farrugia, L. J. Appl. Cryst. 1997, 30, 565





