Crystal Structure Report for 

Prof. Jim Mayer

Submitted by Ian Rhile

Sample ID – IR2224

by Antonio DiPasquale

X-ray Crystallography Laboratory

University of Washington

08. June 2004
Data backup accessibly through ftp:

computer: 

chmraypc.cpac.washington.edu

login: 

anonymous

password:

(your name or email address)

location: 
c:\structures\mayer\ian\IR2224

Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

A.DiPasquale:(206) 616-4212

(x-ray lab):     (206) 543-0210 

Email:
adipasqu@u.washington.edu
An orange crystal prism 0.10 × 0.04 × 0.04 mm in size was mounted on a glass capillary with oil.  Data was collected at -143oC with three sets of exposures as follows:

Scan 

Type
Scanlength (deg.)  start (deg.):(     
      (
         (     
 ( 

1
      
(        
182.0


          
 3.703   -24.500   160.000     0.000      

2

ω
71.5



 3.703 -145.904  -190.073   74.141

Crystal-to-detector distance was 30 mm and exposure time was 10 seconds per degree for all sets.  The scan width was 2.0o.   Data collection was 99.3% complete to 25o in (.  A total of 18,161 partial and complete reflections were collected covering the indices, h = -14 to 14, k = -19 to 15,

 l = -18 to 18.  4,982 reflections were symmetry independent and the Rint = 0.0649 indicated that the data was of slightly better than average quality (0.07). 

Indexing and unit cell refinement indicated a monoclinic P lattice.  The space group was found to be P 21/c (No.14). 

The data was integrated and scaled using hkl-2000. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor, which is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR-97) produced a complete heavy atom phasing model consistent with the proposed structure.  The asymmetric unit contains the target molecule as well as one water molecule which participates in hydrogen bonding with the target molecule.

All hydrogen atoms, with the exception of hydroxyl and amine hydrogens, were placed using a riding model. H(1), H(1a) were located from the Fourier difference map and their locations refined isotropically.  All non-hydrogen atoms were refined anisotropically by full-matrix least-squares (SHELXL-97).




_1097643391.unknown

