Crystal Structure Report for 

Prof. James M. Mayer

Submitted by Ian Rhile

Sample ID – IR3034

by Antonio DiPasquale

X-ray Crystallography Laboratory

University of Washington

29. November 2004
Data backup accessibly through ftp:

computer: 

chmraypc.cpac.washington.edu

login: 

anonymous

password:

(your name or email address)

location: 
c:\structures\mayer\ian\IR3034

Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures/mayer/ian/IR3034/IR3034.htm
Username: 
xray-user

Password: 
hkl-data

A.DiPasquale:(206) 616-4212

(x-ray lab):     (206) 543-0210 

Email:
adipasqu@u.washington.edu
A colorless crystal prism 0.48 × 0.36 × 0.19 mm in size was mounted on a glass capillary with oil.  Data was collected at -143oC with three sets of exposures as follows:

Scan 

Type
Scanlength (deg.)  start (deg.):(     
      (
         (     
 ( 

1
      
(        
181.6


          
 3.703   -60.297   160.000     0.000      

2

ω
56.5



 3.703    94.096  -200.073   74.141

3

ω         26.5



 3.703
 -25.904  -180.073   74.141

Crystal-to-detector distance was 30 mm and exposure time was 10 seconds per degree for all sets.  The scan width was 2.0o.   Data collection was 99.6% complete to 25o in (.  A total of 11,291 partial and complete reflections were collected covering the indices, h = -12 to 12, k = -33 to 35,

 l = -12 to 12.  3,114 reflections were symmetry independent and the Rint = 0.0532 indicated that the data was of better than average quality (0.07). 

Indexing and unit cell refinement indicated a monoclinic C lattice.  The space group was found to be C c (No.9). 

The data was integrated and scaled using hkl-2000. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor, which is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR-97) produced a complete heavy atom phasing model consistent with the proposed structure. 

All hydrogen atoms, with the exception of the hydroxyl, were placed using a riding model. H(1) was located from the Fourier difference map and its locations refined isotropically. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares (SHELXL-97).




_1097643391.unknown

