Crystal Structure Report for 
Prof. Jim Mayer & Prof. Forrest Michael
Submitted by Jessica Hoover
Sample ID – JMHI061
by Antonio DiPasquale & W. Kaminsky
X-ray Crystallography Laboratory

University of Washington

29. July 2005
Data backup accessibly through ftp:

computer: 

chmraypc.cpac.washington.edu

login: 

anonymous

password:

(your name or email address)

location: 
c:\structures\mayer\hoover\JMHI061
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

A.DiPasquale:(206) 616-4212

(x-ray lab):     (206) 543-0210 

Email:
adipasqu@u.washington.edu
A brown needle 0.24 × 0.07 × 0.07 mm in size was mounted on a glass capillary with oil.  Data was collected at -143oC with four sets of exposures as follows:

Scan 

Type
Scanlength (deg.)  start (deg.):(     
      (
         (     
 ( 

1
      
(        
183.0


          
 3.703     -1.500    160.000     0.000      

2

ω
91.5



 3.703     94.096  -200.073   74.141
3

ω
71.5



 3.703    -25.904  -200.073   74.141
4

ω
56.5



 3.703  -145.904  -180.073   74.141

Crystal-to-detector distance was 30 mm and exposure time was 30 seconds per degree for all sets.  The scan width was 2.0o.   Data collection was 98.8% complete to 25o in (.  A total of 10,246 partial and complete reflections were collected covering the indices, h = -11 to 12, k = -11 to 12,

 l = -12 to 13.  3,554 reflections were symmetry independent and the Rint = 0.0778 indicated that the data was of slightly less than average quality (0.07). 

Indexing and unit cell refinement indicated a triclinic P lattice.  The space group was found to be P -1 (No.2). 

The data was integrated and scaled using hkl-2000. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor, which is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR-2004) produced a complete heavy atom phasing model consistent with the proposed structure. 

All hydrogen atoms, with the exception of the amine hygrogen H1, were placed using a riding model. The amine hydrogen H1 was located from the Fourier difference map and its position was refined isotropically.  All non-hydrogen atoms were refined anisotropically by full-matrix least-squares (SHELXL-97).  The asymmetric unit contains half of the target complex, of which the other half of the target complex is generated by symmetry to give the dimer as the full unit cell.



_1097643391.unknown

