Crystal Structure Report for L Dalton

Submitted by Brenden Carlsson 

Sample ID – OsTMEBBYCNPh

by Werner Kaminsky

X-ray Crystallography Laboratory

University of Washington

21. May 2004
Data backup accessibly through ftp:

computer: 

chmraypc.cpac.washington.edu

login: 

anonymous

password:

(your name or email address)

location: 
c:\structures\dalton\brenden\OsTMEBBYCNPh

Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210 

Email: kaminsky@wintensor.com
Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A red crystal plate cut down to 0.20 x 0.24 x 0.12 mm was mounted on a glass capillary with oil.  Data was collected at -143oC with two sets of exposure as follows:

Scan 

Type
Scanlength (deg.)  start (deg.)
(     
(
(     
( 

1
      
(        
138.6




3.70
106.7
160  
 0 

2
      
(        
26.5




3.70
-25.9
-150.1
 74.1           

Crystal-to-detector distance was 30 mm and exposure time was 50 seconds per degree for all sets.  The scan width was 0.9o.   Data collection was 78.5% complete to 29.98o and 94.5% complete to 25o  in (.  A total of  71074 partial and complete reflections were collected covering the indices, h = -15 to 15, k = -23 to 23,  l = -38 to 38. Merged number of reflections was 17466 and 10320 reflections were symmetry independent with an Rint = 0.0655 indicating that the data was good (average quality (0.07). 

Indexing and unit cell refinement indicated a monoclinic P lattice.  The space group was found to be P 21/c  (No.14). 

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S = exp(2B(sinq/()2)/ scale

S is calculated from the scale and the B factor is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. 

All hydrogen atoms were located using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares. 

In addition to the osmium complex we found two chloroform molecules. A single PF6  balances the positive charge of the complex.







