Crystal Structure Report for 
Prof. Jim Mayer
Submitted by Todd Markle
Sample ID – tfm1-078
by Antonio DiPasquale
X-ray Crystallography Laboratory

University of Washington

06. October 2005
Data backup accessibly through ftp:

computer: 

chmraypc.cpac.washington.edu

login: 

anonymous

password:

(your name or email address)

location: 
c:\structures\mayer\markle\tfm1-078
Data backup accessibly through http:

URL: 
http://cad4.cpac.washington.edu/structures
Username: 
xray-user

Password: 
hkl-data

A.DiPasquale:(206) 616-4212

(x-ray lab):     (206) 543-0210 

Email:
adipasqu@u.washington.edu
A colorless block cut to 0.24 × 0.24 × 0.12 mm in size was mounted on a glass capillary with oil.  Data was collected at -143oC with two sets of exposures as follows:

Scan 

Type
Scanlength (deg.)  start (deg.):(     
      (
         (     
 ( 

1
      
(        
182.0


          
3.703   -72.500    160.000     0.000      

2

ω
9.9



3.703   145.904  -159.995  -74.141
Crystal-to-detector distance was 30.0 mm and exposure time was 20 seconds per degree for all sets.  The scan width was 2.0o.   Data collection was 99.7% complete to 25o in (.  A total of 5,769 partial and complete reflections were collected covering the indices, h = -21 to 22, k = -8 to 7, l = -17 to 17.  1,796 reflections were symmetry independent and the Rint = 0.0673 indicated that the data was of slightly better than average quality (0.07). 

Indexing and unit cell refinement indicated a monoclinic C lattice.  The space group was found to be C 2/m (No.12). 

The data was integrated and scaled using hkl-2000. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:


[image: image1.wmf]22

2(sin)/

S = ()/scale

B

e

ql

-


S is calculated from the scale and the B factor, which is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR-2004) produced a complete heavy atom phasing model consistent with the proposed structure. 

All hydrogen atoms, with the exception of the hydroxyl proton, were placed using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares (SHELXL-97).  H1 was located from the difference map and its position was refined isotropically.
The phenol and pyridine rings both lie on the same, crystallographicly required mirror plane.  H1 lies just off that plane and its position is split equally (50% occupancy in each position).  No intermolecular hydrogen bonding is observed in the unit cell packing diagram.  



_1097643391.unknown

